EELECTRON

ITEM SPECIFICATIONS

update 10-05-2019

|  |  | SYSTEM COMPONENTS |  |  |
| --- | --- | --- | --- | --- |
| pcs | 1 | PS00D03KNX | KNX Power supply unit 640 mA - AC 180 ‐ 264 V, 50‐60 Hz DIN module that provides the system power necessary for the KNX bus, with integrated choke to decouple the power supply voltage from the bus. 1 reset button and 3 LEDs (power on, reset, overload). Mounting: 3 DIN modules. Connection to the bus through WAGO clamp. Input voltage: AC 180 ‐ 264 V, 50‐60 Hz; Output voltage: DC 29 V; Output current: max. 640 mA. MANUFACTURER: EELECTRON Art. PS00D03KNX |  |
| pcs | 1 | PS00E03KNX | KNX Power supply unit 640 mA - AC 100 ‐ 240 V, 50‐60 Hz DIN module that provides the system power necessary for the KNX bus, with integrated choke to decouple the power supply voltage from the bus. 1 reset button and 4 LEDs (overvoltage, overload, operation, reset). Mounting: 4 DIN modules. Connection to the bus through WAGO clamp. Input voltage: AC 100 ‐ 240 V, 50‐60 Hz; Output voltage: DC 29 V; Output current: max. 640 mA. MANUFACTURER: EELECTRON Art. PS00E03KNX |  |
| pcs | 1 | LC00B01KNX | Line Coupler / Line Repeater KNX device for DIN rail (1 modules) with the function of line coupler in order to connect a line to a main line or as a backbone coupler to connect a main line to a backbone line. The device has a filter table (8k bytes) and ensures a galvanic isolation between the lines. The coupler supports KNX long frames and is compatible with the ETS® software (ETS 4.2 or higher). The buttons on the front panel allow disabling the telegram filter for testing purposes. The LEDs indicate operating conditions as well as communication errors on the KNX bus. Mounting: 1 DIN module. Controls and indicators: 2 buttons and 3 LEDs multicolors. Supply: KNX main line approx. 5mA; KNX sub line approx. 3mA . Connection to the bus through WAGO clamp. MANUFACTURER: EELECTRON Art. LC00B01KNX |  |
|  |  | CABLES |  |  |
| m | 100 | CV00A02KNX | KNX bus cable 2 x 2 x 0,8 mm Cable for the connection between the devices in a KNX network. It is composed by two couples of twisted cables (red/black and white/yellow). Diameter: 0,8mm. Diameter (cable sheathing included): 6,1mm. 100 m cable hanks. MANUFACTURER: EELECTRON Art. CV00A02KNX |  |
| m | 500 | CV05A02KNX | KNX bus cable 2 x 2 x 0,8 mm Cable for the connection between the devices in a KNX network. It is composed by two couples of twisted cables (red/black and white/yellow). Diameter: 0,8mm. Diameter (cable sheathing included): 6,1mm. 500 m cable hanks. Programming and commissioning are excluded. MANUFACTURER: EELECTRON Art. CV05A02KNX |  |
| m | 100 | CV00A01KNX | KNX bus cable 1 x 2 x 0,8 mm Cable for the connection between the devices in a KNX network. It is composed by a couple of twisted cables (red/black). Diameter: 0,8mm. Diameter (cable sheathing included): 6,1mm. 100 m cable hanks. MANUFACTURER: EELECTRON Art. CV00A01KNX |  |
| m | 500 | CV05A01KNX | KNX bus cable 1 x 2 x 0,8 mm Cable for the connection between the devices in a KNX network. It is composed by a couple of twisted cables (red/black). Diameter: 0,8mm. Diameter (cable sheathing included): 6,1mm. 500 m cable hanks. MANUFACTURER: EELECTRON Art. CV05A01KNX |  |
|  |  | ACCESSORIES |  |  |
| pcs |  | LD00AxxACC | Miniature LED Lamps Packages of 20 or 60 pcs with blue or white light 3V wired red \ black |  |
| pcs | 10 | WG00A01ACC | KNX WAGO connector (red/black) Bus Connector Red/Black for EIB/KNX applications, with direct plug connection. Can connect up to 4 pairs of conductors to a KNX device, it can also be used as a branch terminal. It is composed by two parts connected, in red (“+”) and black (“‐“), with 4 plug‐in terminal each. For wires of about 0,6 and 0,8mm in diameter. Programming and commissioning are excluded. MANUFACTURER: EELECTRON Art. WG00A01ACC (10pcs) |  |
| pcs | 100 | WG00A02ACC | KNX WAGO connector (red/black) Bus Connector Red/Black for EIB/KNX applications, with direct plug connection. Can connect up to 4 pairs of conductors to a KNX device, it can also be used as a branch terminal. It is composed by two parts connected, in red (“+”) and black (“‐“), with 4 plug‐in terminal each. For wires of about 0,6 and 0,8mm in diameter. Programming and commissioning are excluded. MANUFACTURER: EELECTRON Art. WG00A02ACC (100pcs) |  |
|  |  | INTERFACES |  |  |
| pcs | 1 | IN00A03USB | USB - KNX Interface KNX device (DIN Rail – 1 module) with the function of establishing a bidirectional connection between a PC and the EIB/KNX installation bus. Programming and commissioning are excluded. MANUFACTURER: EELECTRON Art. IN00A03USB |  |
| pcs | 1 | IN00A03IPI | KNX - IP Interface KNX device (DIN Rail – 1 module) for direct or LAN connection for programming or supervision of the KNX system. The IP address can be obtained by a DHCP server or by manual configuration (ETS) respectively. The device can also be used as communication interface between LAN (IP) line and KNX bus. KNX bus is connected via WAGO connector. Programming and commissioning are excluded. MANUFACTURER: EELECTRON Art. IN00A03IPI |  |
| pcs | 1 | IN00A02RIP | KNX - IP ROUTER interface KNX device (DIN Rail – 1 module) that is used to allow the forwarding of telegrams between different lines through a LAN (IP) as a fast backbone. The device can also be used as a programming interface KNX bus system. The IP address can be obtained by a DHCP server or by manual configuration (ETS) respectively. KNX bus is connected via WAGO connector, LAN via RJ-45 socket. Programming and commissioning are excluded. MANUFACTURER: EELECTRON Art. IN00A02RIP |  |
| pcs | 1 | IC00R01KNX | KNX ‐ RASPBERRY INTERFACE KNX ‐ Raspberry PI® gateway. This easy to use board can be intend as a gateway developed with a simple serial protocol to receive and send telegrams on the KNX Bus. The on board micro controller includes the KNX Stack required to interact with the KNX Bus. IC00R01KNX is designed to fit exactly the Raspberry GPIO connector. Programming and commissioning are excluded. MANUFACTURER: EELECTRON – ART. IC00R01KNX |  |
|  |  | GATEWAYS |  |  |
| pcs | 1 | IC00B01DMX | KNX -DMX 512 GATEWAY KNX‐DMX Gateway is an interface between the KNX bus and the DMX bus. It combines elements of building automation with a multitude of lighting and special effects devices for every need. One-way gateway that receives data telegrams on the KNX bus and transmits the data onto the DMX512 bus. The KNX‐DMX Gateway contains 8 adjustable sequence positions within the whole channel range. Individual scenes with all 512 channels can be saved and retrieved using KNX group addresses. This function allows all channels to be connected simultaneously to one single object without causing any undo load on the KNX bus. Mounting: DIN rail - 6 modules. Supply: 9-30 V DC, 100mA, separated. Programming and commissioning are excluded. MANUFACTURER: EELECTRON Art. IC00B01DMX |  |
| pcs | 1 | IC00P01DAL | KNX-DALI GATEWAY KNX device DIN rail mounted (4 modules) working as a gateway between KNX installation and digital lighting in DALI, that allows the commutation and dimmable lighting management up to 64 DALI lamps or DALI ballast. It is also possible to use 6 different methods of addressing that allow to send individual or group command by KNX telegram. Up to 32 independent DALI groups are available for the group addressing or 64 individual address through the 64 device’s channels. The device is fully powered through mains voltage and makes the DALI system voltage (typ. DC 16 V) available. Programming and commissioning are excluded. MANUFACTURER: EELECTRON – ART. IC00P01DAL |  |
| pcs | 1 | IC00P02DAL | KNX-DALI TUNABLE WHITE GATEWAY The DALI Gateway is an interface between a KNX installation and DALI lighting system. The DALI gateway allows the switching and dimming of a maximum of 64 lights with a DALI operating device (electronic ballast). Up to 6 different addressing types of the DALI gateway allow group-orientated and individually-addresses control of DALI lights via KNX telegrams. Depending on the configuration, up to 32 independent DALI groups are available for group addressing. For alternative control, these can be supplemented with 64 individually-addressable DALI device channels, as necessary. Optionally, master control of all connected DALI components is possible (broadcast). This means that there is not need to commission DALI, the lighting systems with few functions can be started up quickly and easily (simplified configuration without DALI commissioning). The DALI gateway is supplied completely via the mains voltage connection and makes the DALI system voltage (typ. DC 16V) available. Mounting: DIN rail - 4 modules. Programming and commissioning are excluded. MANUFACTURER: EELECTRON – ART. IC00P02DAL |  |
|  |  | PUSH BUTTONS INTERFACES and ACCESSORIES |  |  |
| pcs | 1 | IO22D01KNX | PUSH BUTTON INTERFACE KNX inwall 2 IN / 2 led OUT module KNX device dedicated to interface dry contacts through the 2 input channels, such as sensors, traditional push buttons, etc. and 2 output channels at low voltage to drive signal lamp or LED e.g. synoptic panels used for monitoring the installation. Extremely compact size: only 34 x 34 x 11 mm. There are also 8 blocks of logic functions freely configurable by ETS. Inputs are configured for the most common applications, e.g. ON/OFF, dimming, shutters and scenarios memo. Device is equipped with KNX communication interface. Mounting: flush mounted wall box. Programming and commissioning are excluded. MANUFACTURER: EELECTRON Art. IO22D01KNX |  |
| pcs | 1 | IO44D01KNX | PUSH BUTTON INTERFACE KNX inwall 4 IN / 4 led OUT module KNX device dedicated to interface dry contacts through the 4 input channels, such as sensors, traditional push buttons, etc. and 4 output channels at low voltage to drive signal lamp or LED e.g. synoptic panels used for monitoring the installation. Extremely compact size: only 34 x 34 x 11 mm. There are also 8 blocks of logic functions freely configurable by ETS. Inputs are configured for the most common applications, e.g. ON/OFF, dimming, shutters and scenarios memo. Device is equipped with KNX communication interface. Mounting: flush mounted wall box. Programming and commissioning are excluded. MANUFACTURER: EELECTRON Art. IO44D01KNX |  |
| pcs | 1 | IO62D01KNX | PUSH BUTTON INTERFACE KNX inwall 6 IN / 2 led OUT module KNX device dedicated to interface dry contacts through the 6 input channels, such as sensors, traditional push buttons, etc. and 2 output channels at low voltage to drive signal lamp or LED e.g. synoptic panels used for monitoring the installation. Extremely compact size: only 34 x 34 x 11 mm. There are also 6 blocks of logic functions freely configurable by ETS. Inputs are configured for the most common applications, e.g. ON/OFF, dimming, shutters and scenarios memo. Device is equipped with KNX communication interface. Mounting: flush mounted wall box. Programming and commissioning are excluded. MANUFACTURER: EELECTRON Art. IO62D01KNX |  |
| pcs | 1 | AD84C01KNX | Push Button interface KNX inwall 8 IN / 4 led OUT module KNX inwall module that includes 4 digital inputs to interface free potential contacts; 4 analog/digital inputs for free potential contacts or temperature sensors and 4 LED outputs. Digital inputs can interface sensors, traditional buttons, etc; 4 LED output channels at low voltage can drive LED for synoptics panels or switches. Inputs 5÷8, set as analog inputs, can enable up to 4 temperature probes (with ON/OFF threshold) and 4 thermostats to control heating and cooling equipments, valves, 2 and 4 pipes fan coil, etc. There are also 5 blocks of logic functions freely configurable by ETS. The device is equipped with appropriate communication interface with the bus type TP1 (twisted pair) KNX European standard, according CEI EN 50090. Programming and commissioning are excluded. MANUFACTURER: EELECTRON Art. AD84C01KNX |  |
| pcs | 1 | TS01A01ACC | KNX temperature probe for internal use Temperature probe for internal use. Use with KNX device AD84A01KNX. Programming and commissioning are excluded. MANUFACTURER: EELECTRON – ART. TS01A01ACC |  |
| pcs | 1 | TS01B01ACC | KNX temperature probe for external use Temperature probe for external use. Use with KNX device AD84A01KNX. Programming and commissioning are excluded. MANUFACTURER: EELECTRON – ART. TS01B01ACC |  |
| pcs | 1 | IR00A01ACC | KNX Infrared receiver for IO32D01KNX KNX infrared receiver for KNX IO32D01KNX, for actuations through remote control. Programming and commissioning are excluded. MANUFACTURER: EELECTRON – ART. IR00A01ACC |  |
| pcs | 1 | RC80A01IRC | Infrared remote control Infrared remote control. Programming and commissioning are excluded. MANUFACTURER: EELECTRON – ART. RC80A01IRC |  |
|  |  | INPUTS - OUTPUTS |  |  |
| pcs | 1 | IO44B02KNX‐C | KNX DIN rail actuator 4 IN / 4 OUT KNX device (DIN RAIL ‐ 4 modules) that is used to interface commands (e.g. push buttons) or loads (e.g. lamps) for any kind of applications. The device is equipped with 4 binary inputs (potential free) and 4 binary relay outputs. Inputs can be connected to conventional switching devices, e.g. push buttons, switches, floating contacts, for switching functions with pulse edge evaluation (e.g. rising or falling edge, toggle, etc...). Outputs include switching function, scene recall, control logic function; timing function and a delay on activation or deactivation can be enabled. Outputs can act as interlocked channels; this function can be used to drive fan coils with 2‐pipe / 3 speed or 4‐pipe / 2 speed. Device is intended to be installed on DIN rail. Programming and commissioning are excluded. MANUFACTURER: EELECTRON Art. IO44B02KNX‐C |  |
| pcs | 1 | IO44E01KNX | KNX DIN rail universal actuator 4 IN / 4 OUT KNX device (DIN RAIL ‐ 4 modules) that is used to interface commands (e.g. push buttons) or loads (e.g. lamps) for any kind of applications. The device is equipped with 4 binary inputs (potential free) and 4 binary relay outputs. Inputs can be connected to conventional switching devices, e.g. push buttons, switches, floating contacts, for switching functions with pulse edge evaluation (e.g. rising or falling edge, toggle...). Inputs can be configured with ETS SW, as output to drive Leds. Inputs can be used to for on/off commands, dimming, shutter control, scene recall and control; outputs include switching function, scene recall and control logic function. 4 outputs on board can be configured: • Each output can be configured independently for load control (R1 to R4) • Each output can be configured independently for ON / OFF or continuous switching (PWM) for electric valves (solenoid actuators) (EV1 to EV4) - Outputs can be configured in pairs for the management of roller shutters and blinds; up to 2 channels (Channels A to B) • Outputs can be configured in pairs for management of motor reductor or for solenoid valves with 3‐point control or for ventilating grille; up to 2 channels (Channels A to B). Programming and commissioning are excluded. MANUFACTURER: EELECTRON Art. IO44E01KNX |  |
| pcs | 1 | IO32D01KNX | KNX inwall actuator 3 IN /2 OUT KNX inwall module that includes: • 2 digital inputs • 1 analog input • 2 relay output (bistable). Digital inputs are intended to be connected to free potential contacts and can interface sensors, traditional buttons, etc; they can be used to on/off commands, dimming, shutter control, scene recall and control, sequences of 3 objects. Analog input, can manage one temperature probe (with On/Off threshold) or one thermostats to control heating and cooling equipments, valves, 2 and 4 pipes fan coils; etc.. Analog input, alternatively to the temperature sensor, can manage an Infrared Receiver (IRX) in order to forward to the bus up to 8 channel coming from an Infrared Remote Control (IRC) with on/off commands, scenes, sequences of 2 objects, dimmer and shutter. Outputs include switching function with timed delays, staircase function, scene recall, lock or logic function. • Each output can be configured independently for load control (2 independent channels) Device 2 outputs on board can be configured: • Each output can be configured independently for ON / OFF or continuous switching (PWM) for electric valves (solenoid actuators) (2 independent channels) • Outputs can be configured in pairs for the management of roller shutters and blinds; (1 channel). The device is equipped with KNX communication interface. Programming and commissioning are excluded. MANUFACTURER: EELECTRON Art. IO32D01KNX |  |
| pcs | 1 | IO88B02KNX‐C | KNX DIN rail actuator 8 IN / 8 OUT KNX device (DIN RAIL ‐ 4 modules) that is used to interface commands (e.g. push buttons) or loads (e.g. lamps) for any kind of applications. The device is equipped with 8 binary inputs (potential free) and 8 binary relay outputs. Inputs can be connected to conventional switching devices, e.g. push buttons, switches, floating contacts, for switching functions with pulse edge evaluation (e.g. rising or falling edge, toggle, etc...). Inputs can be configured with ETS SW, as output channels to drive LEDs in synoptic monitoring panels. Inputs can be used to for on/off commands, dimming, shutter control, scene recall and control; outputs include switching function, scene recall and control logic function. Relay from 5 to 8 are equipped with tab and can be switched manually. Mounting on DIN rail. Programming and commissioning are excluded. MANUFACTURER: EELECTRON Art. IO88B02KNX‐C |  |
| pcs | 1 | IO88E01KNX | KNX DIN rail universal actuator 8 IN / 8 OUT KNX device (DIN RAIL – 4 modules) that is used to interface commands (e.g. push buttons) or loads (e.g. lamps) for any kind of applications. The device is equipped with 8 binary inputs (potential free) and 8 binary relay outputs. Inputs can be connected to conventional switching devices, e.g. push buttons, switches, floating contacts, for switching functions with pulse edge evaluation (e.g. rising or falling edge, toggle...). Inputs can be configured with ETS SW, as output to drive Leds. Inputs can be used to for on/off commands, dimming, shutter control, scene recall and control; outputs include switching function, scene recall and control logic function. Device 8 outputs on board can be configured: • Each output can be configured independently for load control (R1 to R8) • Each output can be configured independently for ON / OFF or continuous switching (PWM) for electric valves (solenoid actuators) (EV1 to EV8) - Outputs can be configured in pairs for the management of roller shutters and blinds; up to 4 channels (Channels A to D) • Outputs can be configured in pairs for management of motor reductor or for solenoid valves with 3‐point control or for ventilating grille; up to 4 channels (Channels A to D) • Fan Coil Actuator for 2/4 pipes systems for Heating / Cooling with 3 speed motors (uses relay from 1 to 5). Programming and commissioning are excluded. MANUFACTURER: EELECTRON Art. IO88E01KNX |  |
| pcs | 1 | IO16F01KNX IO16F01KNX-SD | KNX DIN rail universal actuator 16 IN / 16 OUT The DIN rail 16 input / 16 output module IO16F01NKX is a KNX DIN rail mounting device (8 DIN modules). Device has 16 inputs (potential free) and 16 relays outputs 16A - 230V AC. Input are for dry contacts (potential free) and can be connected to push buttons, switches or as output for leds. Inputs can be used for on/off commands, dimming, shutters control, scenes, sequences, step by step on/off. 4 inputs are configurable as analogical to be connected to NTC temperature probes and achieve up to 4 temperature sensor modules. Outputs can be configured for: - up to 16 independent load/light controls - up to independent PWM control electric valves (solenoid actuators) - up to 8 independent shutter/venetian - up to 8 independent solenoid valves with 3-point control (or ventilating grille) - up to 4 independent 2 pipes fancoil actuators - up to 2 independent 4 pipes fancoil actuators Device includes manual push button for local relays switching and leds for operation indication. Version BO16F01KNX-SD can manage a MicroSD card to save the ETS configuration for fast programming of a new device. MANUFACTURER: EELECTRON – ART. IO16F01KNX / ART. IO16F01KNX - SD |  |
|  |  | OUTPUTS |  |  |
| pcs | 1 | BO04A01KNX | KNX DIN rail universal actuator 4 OUT BO04A01KNX is a KNX DIN rail mounting device ( 4 modules) useful to interface loads (e.g. lamps) for any kind of applications. Device 4 outputs on board can be configured: • Each output can be configured independently for load control (R1 to R4) • Each output can be configured independently for ON / OFF or continuous switching (PWM) for electric valves (solenoid actuators) (EV1 to EV4) - Outputs can be configured in pairs for the management of roller shutters and blinds; up to 2 channels (Channels A to B) • Outputs can be configured in pairs for management of motor reductor or for solenoid valves with 3‐point control or for ventilating grille; up to 2 channels (Channels A to B) Programming and commissioning are excluded. MANUFACTURER: EELECTRON – ART. BO04A01KNX |  |
| pcs | 1 | BO08A01KNX | KNX DIN rail actuator 8 OUT BO08A01KNX is a KNX DIN rail mounting device ( 4 modules) useful to interface loads (e.g. lamps) for any kind of applications. Device 8 outputs on board can be configured: • Each output can be configured independently for load control (R1 to R8) • Each output can be configured independently for ON / OFF or continuous switching (PWM) for Electric valves (solenoid actuators) (EV1 to EV8) - Outputs can be configured in pairs for the management of roller shutters and blinds; up to 4 channels (Channels A to D) • Outputs can be configured in pairs for management of Motor Reductor or for solenoid valves with 3‐point control or for ventilating grille; up to 4 channels (Channels A to D) • Fan Coil Actuator for 2/4 pipes systems for Heating / Cooling with 3 speed motors) (uses relay from 1 to 5). Programming and commissioning are excluded. MANUFACTURER: EELECTRON – ART. BO08A01KNX |  |
| pcs | 1 | BO04B01KNX | KNX DIN rail universal actuator 4 OUT - with manual control BO04B01KNX is a KNX DIN rail ( 4 modules) universal actuator with 4 output 16A and with manual control. It can be used to control up to 4 independent loads / lights, control up to 2 independent blind / roller shutters with mechanical end position. Programming and commissioning are excluded. MANUFACTURER: EELECTRON – ART. BO04B01KNX |  |
| pcs | 1 | BO08B01KNX | KNX DIN rail actuator 8 OUT - with manual control BO08B01KNX is a KNX DIN rail ( 4 modules) universal actuator with 8 output 16A and with manual control. It can be used to control up to 8 independent loads / lights, control up to 4 independent blind / roller shutters with mechanical end position. Programming and commissioning are excluded. MANUFACTURER: EELECTRON – ART. BO08B01KNX |  |
| pcs | 1 | BO12B01KNX | KNX DIN rail actuator 12 OUT - with manual control BO12A01KNX is a KNX DIN Rail ( 9 modules) actuator with 12 output 16 A and with manual control. It can be used to: • Control up to 12 independent loads / lights • Control up to 6 independent blind / roller shutters with mechanical end position. Device, physical address, parameters and communication objects configurations must be made with ETS. Power supply through bus KNX 21..30 V DC SELV. Programming and commissioning are excluded. MANUFACTURER: EELECTRON – ART. BO12A01KNX |  |
| pcs | 1 | BO16F01KNX BO16F01KNX - SD | KNX DIN rail actuator 16 OUT The DIN rail 16 output module BO16F01NKX is a KNX DIN rail mounting device (8 DIN modules) with 16 relays outputs 16A - 230V AC. Outputs can be configured for: - up to 16 independent load/light controls - up to independent PWM control electric valves (solenoid actuators) - up to 8 independent shutter/venetian - up to 8 independent solenoid valves with 3-point control (or ventilating grille) - up to 4 independent 2 pipes fancoil actuators - up to 2 independent 4 pipes fancoil actuators Device includes manual push button for local relays switching and leds for operation indication. Version BO16F01KNX-SD can manage a MicroSD card to save the ETS configuration for fast programming of a new device. MANUFACTURER: EELECTRON – ART. BO16F01KNX / ART. BO16F01KNX - SD |  |
|  |  | DIMMERS |  |  |
| pcs | 1 | DM03B03KNX | KNX DIN Rail 3 x 300W dimmer module KNX 3 channel Dimmer Module is designed to drive dimmable lighting with KNX bus and allows dimming of incandescent and halogen lamps of 230V. The product can control one (900W), two (600+300 W) or three (3x300W) independent lighting circuits. Mounting: DIN rail 6 modules. 1 button for manual control of each output. Programming and commissioning are excluded. MANUFACTURER: EELECTRON Art. DM03B03KNX |  |
| pcs | 1 | DM04D01KNX | KNX DIN Rail 4OUT DIMMER 1‐10V KNX Dimmer used to control lighting circuits via a 1-10V connection, acting upon remote control dimmers or electronic ballasts. Device is intended to be installed on DIN rail (4 modules). The device configuration for commissioning in terms of physical address, group addresses and parameters is done with ETS (Engineering Tool Software) through a download of the Application Program. Programming and commissioning are excluded. MANUFACTURER: EELECTRON Art. DM04D01KNX |  |
| pcs | 1 | DM04A01KNX | UNIVERSAL DIN DIMMER 4 channels x 300W DM04A01KNX is KNX universal power dimmer 4 channels with automatic identification of load type and settable parameters to optimize control of different lampas like LED, incandescent and halogen, CFL dimmable lights, low voltage lamps with electronic or ferromagnetic transformer. Load control is possible in leading and trailing edge. Mounting: 8 DIN modules. MANUFACTURER: EELECTRON Art. DM04A01KNX |  |
| pcs | 1 | DM02A02KNX | UNIVERSAL DIN DIMMER 2 channels x 300W DM02A02KNX is KNX universal power dimmer 2 channels with automatic identification of load type and settable parameters to optimize control of different lampas like LED, incandescent and halogen, CFL dimmable lights, low voltage lamps with electronic or ferromagnetic transformer. Load control is possible in leading and trailing edge. Mounting: 4 DIN modules. MANUFACTURER: EELECTRON Art. DM02A02KNX |  |
| pcs | 1 | DL04A01KNX | KNX LED DIMMER 4 channels - RGB\White DL04A01KNX is a dimming actuator for LED in DC with constant voltage (CV). The device allows to drive 4 indipendent channels or 1 RGB channel and 1 single color channel or 1 RGBW channel. Module can be powered from 12 to 48V DC and consequently can manage the outputs (LED strips) with voltage from 12 to 48V DC. The device includes a 16A relay, suitable for switching capacitive loads that allows a complete shutdown of the external power supply when all loads are switched off (for example at night). Other available functions: block logics, scenes, color sequences. Mounting: DIN rail - 4 modules. Device is equipped with KNX communication interface. MANUFACTURER: EELECTRON Art. DL04A01KNX |  |
| pcs | 1 | DM01D01KNX | KNX UNIVERSAL DIMMER 1 channel 700W - master KNX power dimmer 1-channel (4 DIN modules) acting as a Master Dimmer to which you can connect up to two Slave Modules (cod. DM01D01ACC) with identical characteristics to the Master power dimmer and connected to it by a local two wires bus. Dimmer DM01D01KNX can be used in trailing edge or in leading edge. The three channels are independent and can therefore operate on different phases of the same three phase systems respecting the limit of 230Vac between phase and neutral. Programming and commissioning are excluded. MANUFACTURER: EELECTRON – ART. DM01D01KNX |  |
| pcs | 1 | DM01D01ACC | KNX UNIVERSAL DIMMER 1 channel 700W - slave MANUFACTURER: EELECTRON – ART. DM01D01ACC |  |
|  |  | FANCOIL CONTROLLER |  |  |
| pcs | 1 | TC17B01KNX | FANCOIL CONTROLLER 0-10V The fancoil unit controller is used to control fancoil units, floor heating or switch actuators. Depending on the design of the device, fancoil units are used in 2-pipe or 4-pipe system. It controls up to 3 fan speed (relay or 0-10V outputs) as well as heating or cooling valves (proportional or electrothermal valve) respectively. The mode of control is based on two-steps control or a time-discrete PI controller with setpoint\actual value comparison. The valves and the fan can be regulated directly by devices via the closed loop of this controller. When the fancoil unit controller is used in floor heating, it can control up to seven channels. All of the floor heating channel control is used a time-discrete PI controller with setpoint\actual value comparison. Mounting: DIN rail 4 modules. Supply from KNX bus. 5 relay outputs 10A 230Vac , 2 analog outputs 0-10V DC 10mA for each channel. MANUFACTURER: EELECTRON Art. TC17B01KNX |  |
| pcs | 1 | TS01F01ACC | Temperature probe for fancoil controller 0-10V MANUFACTURER: EELECTRON Art. TS01F01ACC |  |
|  |  | COMPACT ENERGY METER |  |  |
| pcs | 1 | PM10D01KNX | ENERGY METER SINGLE PHASE 63A - compact - KNX KNX active energy meter that provides the essential measurement capabilities required to monitor a single phase electrical installation. • 0.25-5 (63) A, Class B, 230 VAC 50 Hz, -25 °C … +55 °C, 4 Quadrants, 2 Tariffs. • Active Energy Class B (according to EN-50470) and Reactive Energy Class 2 (according to IEC 62053-23) • Direct connected (up to 63 A) • LCD display and 3 push-button keys (to read Energies, V, I, PF, F, P, Q and to configure some parameters) • 1 push botton and 1 LED dedicated to KNX. • Display with 8 digits. • Self supplied (by the input voltage itself). Mounting: DIN rail 2 modules. MANUFACTURER: EELECTRON Art. PM10D01KNX |  |
| pcs | 1 | PM30D01KNX | Energy meter three phase 63A - compact This Energy-meter provides the essential measurement capabilities required to monitor a three phase electrical installation. • Direct connected (up to 63 A) • LCD display and 3 push-button keys (to read Energies, V, I, PF, F, P, Q and to configure some parameters) • 1 push botton and 1 LED dedicated to KNX. • Display with 8 digits. • Self supplied (by the input voltage itself). Mounting: DIN rail 4 modules. c |  |
| pcs | 1 | PM30D02KNX | Energy meter three phase with TA connection - compact This Energy-meter provides the essential measurement capabilities required to monitor a three phase electrical installation. • TA connection (up to 63 A) • LCD display and 3 push-button keys (to read Energies, V, I, PF, F, P, Q and to configure some parameters) • 1 push botton and 1 LED dedicated to KNX. • Display with 8 digits. • Self supplied (by the input voltage itself). Mounting: DIN rail 4 modules. MANUFACTURER: EELECTRON Art. PM30D02KNX |  |
|  |  | PRESENCE and BRIGHTNESS SENSOR |  |  |
| pcs | 1 | PD00D01KNX | KNX PRESENCE SENSOR AND LUX METER - flush mounted ceiling The device is a presence and motion detection with integrated brightness sensor. The device communicates via KNX bus. It is designed for recessed-ceiling mounting. Dimensions: 58x87x87 mm. Type of presence sensor: passive infrared (PIR) \ 288 sectors for detection. Type of brightness measurement: contrast measurement. MANUFACTURER: EELECTRON Art. PD00D01KNX |  |
| pcs | 1 | PD00D01ACC | Surface mounting enclosure for PD00D01KNX |  |
| pcs | 1 | PD02X01CON | CONVENTIONAL PRESENCE SENSOR - 2 channels Is a ceiling flush mount PIR detector. the load will be switched on automatically when the movement is detected on automatically when the movement is detected and the ambient light level is below the lux setting value. Until there is no movement detected and the pre-set delay time has been expired, load will be switched off automatically. User can pre-set the desidered lux and time values by VR or IR setting for automatic control lighting on-off. PD02X01CON can also be used in many different places for automatic control (lux is invalid). Dimensions: 64x80x80 mm. IP54. MANUFACTURER: EELECTRON Art. PD02X01CON |  |
|  |  | 3,5" TOUCH PANEL KNX |  |  |
| pcs | 1 | VS00ExxKNX | EELECTA - KNX Touch Panel 3,5” KNX Touch panel 3,5” with a couloured display, dimming, status, values, lighting, shutters and timers are controlled and password protected when needed. Using the embedded temperature sensor, chrono or fancoil controlling functions are managed. DMX couloured Led or lights are controlled with the optional DMX interface, and load control with automatic cut off of prioritized functions is performed with the available power meter. Based on Linux® OS but ETS programmed, the 3,5” touch panel has Led indicator for status display and an audio signal for alarm functions and is available in three colours. Mounting: inwall box 2 or 3 modules italian, german box, swiss box. Power Supply: auxiliary supply 9....32V DC; from KNX bus 21....32V DC. Dimension: 112 x 110 x 48 mm. Programming and commissioning are excluded. MANUFACTURER: EELECTRON Art. VS00ExxKNX |  |
| pcs | 1 | VS00xxxKNX | GLASSPAD - KNX Touch Panel 3,5” KNX Touch panel 3,5” with a couloured display, dimming, status, values, lighting, shutters and timers are controlled and password protected when needed. Using the embedded temperature sensor, chrono or fancoil controlling functions are managed. DMX couloured Led or lights are controlled with the optional DMX interface, and load control with automatic cut off of prioritized functions is performed with the available power meter. Based on Linux® OS but ETS programmed, the 3,5” touch panel has Led indicator for status display and an audio signal for alarm functions and is available in three colours. Mounting: inwall box 2 or 3 modules italian, german box, swiss box. Power Supply: auxiliary supply 9....32V DC ; from KNX bus 21....32V DC. Dimension: 113 x 113 x 48 mm. Programming and commissioning are excluded. MANUFACTURER: EELECTRON Art. VS00ExxKNX |  |
| pcs | 1 | VS00xxxKNX | 3025 - KNX Touch Panel 3,5” KNX Touch panel 3,5” with a couloured display, dimming, status, values, lighting, shutters and timers are controlled and password protected when needed. Using the embedded temperature sensor, chrono or fancoil controlling functions are managed. DMX couloured Led or lights are controlled with the optional DMX interface, and load control with automatic cut off of prioritized functions is performed with the available power meter. Based on Linux® OS but ETS programmed, the 3,5” touch panel has Led indicator for status display and an audio signal for alarm functions and is available in three colours. Mounting: inwall box 2 or 3 modules italian, german box, swiss box. Power Supply: auxiliary supply 9....32V DC; from KNX bus 21....32V DC. Dimension: 113 x 113 x 48 mm. Programming and commissioning are excluded. MANUFACTURER: EELECTRON Art. VS00ExxKNX |  |
| pcs | 1 | PS00A11ACC | Power supply 12V - 15W for touch panel KNX DIN rail power supply for 3,5" touch panel: 12 V - 15 W. Up to 6 touch panel. Programming and commissioning are excluded. MANUFACTURER: EELECTRON – ART. PS00A11ACC |  |
| pcs | 1 | PS00A12ACC | Power supply 12V - 24W for touch panel KNX DIN rail power supply for 3,5" touch panel: 12 V - 30 W. Up to 12 touch panel.. Programming and commissioning are excluded. MANUFACTURER: EELECTRON – ART. PS00A12ACC |  |
|  |  | EELECTA - KNX SWITCH |  |  |
| pcs | 1 | PB40AxxCON | Conventional 4 channels Eelecta slave interface Homepad The conventional switch available in 3 colors (White, Silver & Anthracite). Range of switches characterized by a central cross that can be personalized with different finishes and multiform switches cover silk screen printing. The switch can be installed on 2 or 3 module boxes. The product has 4 contacts and must be connected with a KNX communication module. (IO44C02KNX , PB40CxxKNX – PB80BxxKNX). Dimensions: 110x110 mm. Programming and commissioning are excluded. MANUFACTURER: EELECTRON Art. PB40AxxCON |  |
| pcs | 1 | PB40BxxKNX | KNX Switch Homepad 4 channels KNX switch available in 3 colors (White, Silver & Anthracite). Range of switches characterized by a central cross that can be personalized with different finishes and multiform push button cover silk screen printing. The switch has 5 state signaling or night mode Leds, can be install on 2 or 3 module boxes and has 4 channels. The switch pressure can be long or short and can command sequences. Dimensions: 110x110 mm. Programming and commissioning are excluded. MANUFACTURER: EELECTRON Art. PB40BxxKNX |  |
| pcs | 1 | PB40CxxKNX | KNX Switch Homepad 4 channels + 4 IN + Thermostat KNX switch available in 3 colors (White, Silver & Anthracite). Range of switches characterized by a central cross that can be personalized with different finish and multiform switches cover silk screen printing. The switch has 5 state signaling or night mode Leds, can be installed on 2 or 3 module boxes, and has 4 channels. The pressure of push button can be long or short and can command sequences. The switch has a temperature sensor included which can be configured as a room thermostat to manage ON/OFF actuators for conditioning and heating, electric valves zone, 2/4 tube fan coil controlling with 1,2,3 speed. 4 free inputs are available to interface traditional switches from the same range or other conventional switches. (PB40AxxCON). Dimensions: 110x110 mm. Programming and commissioning are excluded. MANUFACTURER: EELECTRON Art. PB40CxxXX |  |
| pcs | 1 | PB80AxxKNX | KNX switch Homepad 8 channels + Thermostat KNX switch available in 3 colors (White, Silver & Anthracite). Range of switches characterized by a central cross that can be personalized with different finishes and multiform push button cover silk screen printing. The switch has 5 state signaling or night mode Leds, can be installed on 2 or 3 module boxes, and has 8 channels. The pressure of push button can be long or short and can command sequences. The switch has a temperature sensor included which can be configured as a room thermostat to manage ON/OFF actuators for conditioning and heating, electric valves zone, 2/4 tube fan coil controlling with 1,2,3 speed. Dimensions: 110x110 mm. Programming and commissioning are excluded. MANUFACTURER: EELECTRON Art. PB80AxxKNX |  |
| pcs | 1 | PB80CxxKNX | KNX switch Homepad 8 channels/4 IN KNX switch available in 3 colors (White, Silver & Anthracite). Range of switches characterized by a central cross that can be personalized with different finishes and multiform push button cover silk screen printing. The switch has 5 state signaling or night mode Leds, can be installed on 2 or 3 module boxes, and has 8 channels. The pressure of push button can be long or short and can command sequences. 4 free inputs are available to interface traditional switches from the same range or other conventional switches. (PB40AxxCON). Dimensions: 110x110 mm. Programming and commissioning are excluded. MANUFACTURER: EELECTRON Art. PB80BxxKNX |  |
| pcs | 1 | MB40BxxKNX | KNX switch Minipad 4 channels KNX switch suitable for inwall boxes 2 modules, with 4 channels characterized by 5 state signaling or night mode Leds. The pressure of push button can be long or short and can command sequences. Power supply: through KNX cable 21..30V DC. Dimensions: 90 x 90 mm. Programming and commissioning are excluded. MANUFACTURER: EELECTRON Art. MB40BxxKNX–WH/CR/BL |  |
| pcs | 1 | MB40CxxKNX | KNX switch Minipad 4 channels + 4 IN + Thermostat KNX switch suitable for inwall boxes 2 modules, with 4 channels characterized by 5 state signaling or night mode Leds. The pressure of push button can be long or short and can command sequences. The switch has a temperature sensor included which can be configured as a room thermostat to manage ON/OFF actuators for conditioning and heating, electric valves zone, 2/4 tube fan coil controlling with 1,2,3 speed. 4 free inputs are available to interface traditional switches from the same range or other conventional switches. (PB40AxxCON). Power supply: through KNX cable 21..30V DC. Dimensions: 90 x 90 mm. Programming and commissioning are excluded. MANUFACTURER: EELECTRON Art. MB40CxxKNX–WH/CR/BL |  |
| pcs | 1 | MB80AxxKNX | KNX Switch Minipad 8 channels + Thermostat KNX switch suitable for inwall boxes 2 modules, with 8 channels characterized by 5 state signaling or night mode Leds. The pressure of push button can be long or short and can command sequences. The switch has a temperature sensor included which can be configured as a room thermostat to manage ON/OFF actuators for conditioning and heating, electric valves zone, 2/4 tube fan coil controlling with 1,2,3 speed. Power supply: through KNX cable 21..30V DC. Dimensions: 90 x 90 mm. Programming and commissioning are excluded. MANUFACTURER: EELECTRON Art. MB80AxxKNX –WH/CR/BL |  |
| pcs | 1 | MB80CxxKNX | KNX Switch Minipad 8 channels + 4 IN KNX switch suitable for inwall boxes 2 modules, with 8 channels characterized by 5 state signaling or night mode Leds. The pressure of push button can be long or short and can command sequences. 4 free inputs are available to interface traditional switches from the same range or other conventional switches. (PB40AxxCON). Power supply: through KNX cable 21..30V DC. Dimensions: 90 x 90 mm. Programming and commissioning are excluded. MANUFACTURER: EELECTRON Art. MB80BxxKNX–WH/CR/BL |  |
|  |  | EELECTA - Z-WAVE SWITCH |  |  |
| pcs | 1 | PB41AxxZWE-xx | Z-Wave Switch Homepad 4 channels + 1 relay Eelecta is a device able to provide the command of a remote dry contact load and the management of 4 commands through Z-Wave technology. 1. Management of a load with dry contact locally: the local dry contact is handled by one of the four buttons chosen during the setup. 2.Load/contacts management remotely: the device has 4 buttons for remote loads control, it’s equipped on board with a relay controllable by one of the four buttons. MANUFACTURER: EELECTRON Art. PB41AxxZWE-xx |  |
|  |  | SYNCHRONICITY |  |  |
| pcs | 1 | TM11CxxKNX | KNX Inwall HVAC thermostat - 1 IN / 1 OUT KNX thermostat for controlling temperature applications in Home & Building Automation, and is characterized by the possibility of mounting on embedding boxes of 2 or 3 modules rectangular or round. It is available in a range of colours for the plastic housing, or glass finishes can be ordered separately, which allows you to match the thermostat to all types of environments. The thermostat features a large LCD for displaying the current temperature or the setpoint, fan speed, summer / winter mode and the 4 operating states. The unit is equipped with a physical ON / OFF input which can be used to interface a sensor for a window or some other device, and an output relay to control the speed 1 of the fan coil or even a zone valve. The thermostat can also be configured for using it in combination with an output ON / OFF module to control the three fan coil speeds. Control elements available to the user include two buttons for increasing or decreasing the setpoint temperature (current setting), and two buttons for increasing or decreasing the fan speed, or stopping the fan altogether. Device configuration, settings, parameters and communication objects can be accomplished with ETS software (Engineering Tool Software). Programming and commissioning are excluded. MANUFACTURER: EELECTRON – ART. TM11CxxKNX |  |
| pcs | 1 | TM10DxxKNX | KNX Inwall HOTEL thermostat - 1IN The Hotel thermostat is a KNX wall mounting device designed for HVAC applications (e.g., office, hotels, hospitals, private houses etc...). The device is equipped with one binary input (potential free contact) that can be used, for instance, to start\stop the HVAC units when a window has been opened (or closed) or for a general purpose usage. The thermostat features a large LCD for displaying the current temperature of the setpoint, fan speed, summer\winter mode and the 4 operating systems. Mounting on embedding boxes of 2 or 3 modules rectangular or round. Programming and commissioning are excluded. MANUFACTURER: EELECTRON – ART. TM10DxxKNX |  |
| pcs | 1 | TR22AxxKNX | KNX Transponder card/tag reader - 2 IN / 2 OUT KNX card reader dedicated to access card control with the transponder; it is customizable versatile and modular, and suitable for almost any application. Mounting boxes are embedded, rectangular or round with 2 or 3 modules. The plastic housing is available in a range of colours, and a glass finishes can be ordered separately, allowing the unit to be matched with every kind of design. The device has two physical ON / OFF inputs available for controlling the door open / close contacts or other signals (window contacts, bathroom pull alarms etc.). The device has two relays for lock management, courtesy light control or another use. On the reader front there are 4 LEDs, 3 of which are freely configurable, and used respectively for the reporting about: access granted, access denied, emergency, required room redo, room occupied or do not disturb. The transponder reading can be made at a distance of 30 mm. The primary feature is the number‐control facility. If the code is consistent, the device will continue by verifying the validity of the "date" field (if enabled). Then the "password" will be checked with the associated codes with it (hours, customer number, service number), and enabled. If verified, the door will open, and (if set), the courtesy light will be turned on, and the reading will be forwarded to the bus. ETS configuration. Programming and commissioning are excluded. MANUFACTURER: EELECTRON – ART. TR22AxxKNX |  |
| pcs | 1 | TH22AxxKNX | KNX Transponder card holder - 2 IN / 2 OUT KNX transponder holder fitted in rectangular boxes with 2 or 3 modules. It can be used for detecting and monitoring the presence of customers or service staff in a room. A range of colours is available for plastic housing, and glass finishes can be ordered separately, and it can be connected to all types of environments. The device has two physical ON / OFF inputs for controlling an open / close door contact or other signals (e.g. window contacts, bathroom alarm pulls, etc.). On the device there are two relays to control the door lock and to control the " courtesy lighting ", or for any other freely configurable use. The front of the transponder is illuminated if no light is available (for dark locations), goes out if the card is invalid, and flashes for 3 seconds if access is not allowed. Card validation is made by checking the following data: ‐"number system", if the code is coherent, it verifies the "date" field (if enabled) to verify validity (if this hasn’t expired). It then checks the "password" with all the associated and enabled codes (time, customer number, service number). If all the conditions matched, then is possible to open the door and, if set, turns on the courtesy light and sends the result to the bus. Configuration via ETS. Programming and commissioning are excluded. MANUFACTURER: EELECTRON – ART. TH22AxxKNX |  |
| pcs | 1 | TR32Ax9KNX | KNX Transponder card/tag reader with bell KNX Transponder Reader (wall mounting device) suitable to access control application. This device can be used in any kind of building (Hotel, Hospital, Offices, Parking, etc..) where the access control application is required. The device is equipped with two binary inputs (potential free contacts) that can be used, for instance, to control whether the door lock has been opened or closed or other signals coming from external switches/contacts (i.e. windows, bathroom emergency alarms, etc..) and with one frontal switch freely configurable by ETS. The transponder reader is equipped also with two output relays which can be used for any purposes, typically to open the door or turning on the courtesy light inside the room. The product provides on the front side four LEDs in order to enlighten 4 icons to display the following states (e.g. in case of Hotel management): - Access Allowed/Not Allowed - SOS request - Service Call (clean room, etc..) - Client status (“Busy room” or “Do not Disturb”) The LEDs and icons can be configured in association with other alarms or events. The transponder reader can reads cards or keys at a maximum distance of 30mm from the front side. The access control follows a sequence where the “build number” has been checked at first, then the “data” to check if it has been elapsed or not, next the password ID for client/ guest/service identification, then the enabled entrance timing window. In case card reading pass all checks the door is opened and, if set, the courtesy light inside is switched on. At the same time the Transponder Holder can send the reading response to the bus. The device configuration for commissioning in terms of physical address, group addresses and parameters is done with ETS ( Engineering Tool Software) through a download of the Application Program. Programming and commissioning are excluded. MANUFACTURER: EELECTRON – ART. TR32AxxKNX |  |
| pcs | 1 | TR22A11KNX‐EXT | KNX Transponder card/tag reader for external mounting IP49 without plexi KNX external card reader dedicated to access control. This product is for both 2 modules round and squared inwall boxes or for 3 modules boxes, the plexiglass front of the product is available in two different colours. The device has two physical binary inputs available for controlling the door open / close contacts or freely configurable by the user (window contacts, bathroom pull alarms etc.). The device has two relays for lock management, courtesy light control or another use. On the reader front there are 4 LEDs, 3 of which are freely configurable, usually programmed to report about: access granted, access denied, emergency, make up room, room occupied or do not disturb. Programming and commissioning are excluded. This article is to be combined with one of the following codes for the different colors: MANUFACTURER: EELECTRON – ART. PX15A14ACC (plexi support frame – black) MANUFACTURER: EELECTRON – ART. PX10A24ACC (plexi support frame – white) |  |
| pcs | 1 | TE00A01KNX | KNX Transponder encoder black The programming card is a transponder that reads / writes transponder cards or items. The device is fitted in a table container with 3 modules, and is equipped with a USB for the connection to a PC. It is back lighted to signal transponder reading or writing. The reader / writer is powered up through the USB port of the PC, which must be provided with the appropriate software to allow the following read/write data: system code, password and date. Programming and commissioning are excluded. MANUFACTURER: EELECTRON – ART. TE00A01KNX (black glass support frame) |  |
| pcs | 1 | CD00A0xTRC | Transponder card – blank Dimensions and characteristics of the Card. Complies with ISO 7810 (85.6 x 54 x 0.76 mm). Possibility of serigraphy on both sides (on request). Dual technology version (RFID and magnetic stripe on request). Programming and commissioning are excluded. MANUFACTURER: EELECTRON – ART. CD00A02TRC (card pack 50 pcs) MANUFACTURER: EELECTRON – ART. CD00A03TRC (card pack 250 pcs) |  |
| pcs | 50 | CD00B02TRC | Transponder keyholder Dimensions and characteristics of the keyholder. Dimensions and materials: ABS 38 x 34 x 6 mm. Frequency: 125 KHZ. Temperature: from -10°C to 50°C. |  |
| pcs | 1 | SW0xD05KNX | EMBEDDED PC WITH eSuite Software – complete, with 2 clients Embedded PC configured to manage access control, thank to the eSuite software pre‐ installed. Software designed for the hotel supervision, KNX settings and for alarms and access control. The software can be interfaced with hotel management software. Remote control and managing via Internet or Ethernet client . Main functions: ‐ The Logs ,after fixed time interval, could be deleted and exported from the Software. ‐ Automatic Check‐out and cancellation of clients after a set time from the departure date. ‐ Replaces the previous “Etask” module, commands are now protected by user login ‐ Service of supervision for monitoring single activities through real time message ‐Direct import from ETS3 of project’s structure and communication objects ‐“Arrange” function added, possibility to inherited properties of objects from a “Master” zone ‐New management client/zone, possibility of checking time delayed ‐ Management on planner of single card or client ‐ Visualization on planner for state of client/service/maintenance, first aid directly in room. ‐ Management on planner for bedroom redo ‐ Data Client archive for recall and automatic export of external rework ‐ Heritability of profiles accesses areas from the existing profiles for client and service ‐ Send automated data on bus check in ‐ Automated function of check out ‐ Function visualization transit :defined a reader for the plant, the passage of the card on this reader show a pop up with the dates of the card and if it previously charged also a photo of the client. ‐ Management mode clients list/table ‐ Management service mode list/table ‐ Management room redo with log updated in real time ‐ Log/events KNX values ‐ KNX graphic values logged ‐ Log in application events, it comes logged the operations from operators or system ‐ Navigator filter for classes area, defined and stores for each clients – Function enabling/edit timers ‐ Simplify and speed management for program card ‐ Recognition/Visualization for the cards data into the encoder. Programming and commissioning are excluded. MANUFACTURER: EELECTRON – ART. SW01D05KNX (Embedded PC with eSuite and 2 clients - up to 10 rooms) MANUFACTURER: EELECTRON – ART. SW02D05KNX (Embedded PC with eSuite and 2 clients - up to 25 rooms) MANUFACTURER: EELECTRON – ART. SW03D05KNX (Embedded PC with eSuite and 2 clients - up to 40 rooms) MANUFACTURER: EELECTRON – ART. SW04D05KNX (Embedded PC with eSuite and 2 clients - up to 100 rooms) MANUFACTURER: EELECTRON – ART. SW05D05KNX (Embedded PC with eSuite and 2 clients - up to 150 rooms) MANUFACTURER: EELECTRON – ART. SW06D05KNX (Embedded PC with eSuite and 2 clients over 150 rooms) |  |
|  |  | WEB SERVER |  |  |
| pcs | 1 | IN00B02WEB | WEB SERVER HORIZONE HORIZONE is a supervising web server for Home & Building Automation based on KNX standards, with the possibility of integrating the Modbus standard and the most widespread technological systems on the market, alarm systems and audio systems / video. Compatible with Mac OS X, Microsoft Windows, Apple iOS and Google Android operating systems, the configuration and use of HORIZONE takes place via web pages using a browser (Safari, Google Chrome, ...), from any type of device ( pc / mac, smartphone and tablet) or using the free app Apple or Android. Mounting: DIN rail 5 modules. Supply: 12....24V DC. Sizes: 200 points / 800 points / 1400 points. MANUFACTURER: EELECTRON – ART. IN00B02WEB (200 points) |  |
|  |  | DOCKING STATION |  |  |
| pcs | 1 | DS00xxxACC | DOCKING STATION for iPad Mini 2 \ iPad Mini 4 \ iPad Air 2 \ iPad Pro 9.7" Elegant wall aluminum solution for iPad and iPad Mini, dedicated to managing the intelligent system. Docking station can hold and charge the iPad, without any visible cables it can power the docking station by an inwall power supply. Lightning / USB cable and power supply 5V - 1A included. MANUFACTURER: EELECTRON – ART. DS00xxxACC |  |
|  |  | 9025 SERIES |  |  |
| pcs | 1 | CS10A01KNX-x | 9025 KNX CAPACITIVE SWITCH The KNX® 9025 series of push buttons consists of 2 - 4 - 6 - 8 - 10 channels capacitive buttons. Each button can be configured to manage on / off commands, dimmers, shutters and blinds, execution and learning of scenarios, sequences of objects, etc. The device includes a 2-stage thermostat with integrated PI controller for driving devices heating and cooling systems, valves, 2 and 4-pipe fan coils, etc ... The device has a temperature probe on board and it has a rear connector (2 poles) configurable as digital or analog input. It's possible to connect a NTC temperature probe (eelectron code TS01A01ACC or TS01B01ACC - not included) to have a direct temperature measurement. The device can include a RGB bar on the front side in order to visualize feedback or other values available over the KNX bus. The glass covers are available in 2 ranges: STANDARD or RGB; for each range the CUSTOM version is available. The glasses in the CUSTOM version have the possibility of backlighting custom and interchangeable icons to be associated with the configured function. The 9025 KNX® series is intended to be used in british box, german box or italian 2 modules box. Device is equipped with KNX communication interface. Dimensions: 96 x 96 mm. MANUFACTURER: EELECTRON – ART. CS10A01KNX-x |  |
| pcs | 1 | Rx07A01KNX-x | 9025 KNX CAPACITIVE thermostat\humidistat The thermostat 9025 is a KNX® controller that includes 7 configurable capacitive buttons for managing on / off commands, dimmers, shutters and blinds, execution and learning of scenarios, object sequences, local thermostat commands, etc. The device includes a 2-stage thermostat with integrated PI controller for driving heating and cooling appliances, valves, 2 and 4-pipe fan coils, etc. The device has a temperature probe on board and it has a rear connector (2 poles) configurable as digital or analog input. It's possible to connect a NTC temperature probe (eelectron code TS01A01ACC or TS01B01ACC - not included) to have a direct temperature measurement. The version with integrated temperature and relative humidity sensor is available, which can be used to control actuators for regulating ambient relative humidity. The device has a RGB bar on the front side in order to visualize feedback or other values available over the KNX bus. The glass covers are available in the version for HOTEL or RESIDENTIAL applications; each also available in the CUSTOM version. The CUSTOM glasses have the possibility of backlighting custom and interchangeable icons that can be associated with the configured function. The KNX® 9025 series is intended to be used in british box, german box or italian 2 modules box. Device is equipped with KNX communication interface. Dimensions: 96 x 96 mm. MANUFACTURER: EELECTRON – ART. Rx07A01KNX-x |  |
|  |  | 3025 KNX SWITCH |  |  |
| pcs | 1 | SB40AxxKNX-xxxx | 3025 KNX SWITCH 4 ch. + thermostat 55x55 mm SB40AxxKNX-xxxx is a KNX tactile 4 channels push button wich can be configured to manage on/off commands, dimming, shutters, and venetians control, scene recall and control, sequences of 3 objects, etc... Device includes a 2 stage Room Temperature Controller with integrated PI to control heating and cooling equipments, valves, 2 and 4 pipes fan coils, etc... Device has a rear connector (2 poles) configurable as digital or analog input. It's possible to connect a NTC temperature probe (eelectron code TS01A01ACC or TS01B01ACC - not included) to have a direct temperature measurement. SB40AxxKNX-xxxx has a RGB bar on the front side in order to visualize feedback or other values available over the KNX bus. SB40AxxKNX-xxxx is intended to be used in british box, german box or italian 2 modules box. Device is equipped with KNX communication interface. Order codes are referred only to switches without frames. MANUFACTURER: EELECTRON – ART. SB40AxxKNX-xxxx |  |
|  |  | 3025 Z-WAVE SWITCH |  |  |
| pcs | 1 | SB41AxxZWE-xxxx | 3025 Z-Wave switch 4 ch. + 1 relay 55x55 mm 3025 is a device able to provide the command of a remote dry contact load and the management of 4 commands through Z-Wave technology. 1. Management of a load with dry contact locally: the local dry contact is handled by one of the four buttons chosen during the setup. 2. Load/contacts management remotely: the device has 4 buttons for remote loads control, it’s equipped on board with a relay controllable by one of the four buttons. Order codes are referred only to switches without frames. MANUFACTURER: EELECTRON – ART. SB41AxxZWE-xxxx |  |
|  |  | GLASSPAD SERIES |  |  |
| pcs | 1 | GP03A03KNX | KNX switch GlassPad KNX 3 CH. / 3 Modules KNX capacitive switch 3 channels available in 2 colours (white and black). Mounting in 3 modules inwall box. The product has led signalling for each channels and can be installed vertically or in horizontal. Pression on the switch can be long or short and it can manage scenarios or sequences. The customization of the glass color, texts and icons for residential and building filed is available on demand. The products if fully programmable with ETS. This device must be combined with glass cover for GlassPad, cod. GL03H03ACC‐1 (horizontal white), GL03H03ACC‐3 (horizontal black), GL03V03ACC‐1 (vertical white), GL03V03ACC‐3 (vertical black). Programming and commissioning are excluded. MANUFACTURER: EELECTRON – ART. GP03A03KNX |  |
| pcs | 1 | GP06A03KNX | KNX switch GlassPad KNX 6 CH. / 3 Modules KNX capacitive switch 6 channels available in 2 colours (white and black). Mounting in 3 modules inwall box. The product has led signalling for each channels and can be installed vertically or in horizontal. Pression on the switch can be long or short and it can manage scenarios or sequences. The customization of the glass color, texts and icons for residential and building filed is available on demand. The products if fully programmable with ETS. This device must be combined with glass cover for GlassPad, cod. GL06H03ACC‐1 (horizontal white), GL06H03ACC‐3 (horizontal black), GL06V03ACC‐1 (vertical white), GL06V03ACC‐3 (vertical black). Programming and commissioning are excluded. MANUFACTURER: EELECTRON – ART. GP06A03KNX |  |
| pcs | 1 | GP04A04KNX | KNX switch GlassPad KNX 4 CH. / 4 Modules KNX capacitive switch 4 channels available in 2 colours (white and black). Mounting in 4 modules inwall box. The product has led signalling for each channels and can be installed vertically or in horizontal. Pression on the switch can be long or short and it can manage scenarios or sequences. The customization of the glass color, texts and icons for residential and building filed is available on demand. The products if fully programmable with ETS. This device must be combined with glass cover for GlassPad, cod. GL04H04ACC‐1 (horizontal white), GL04H04ACC‐3 (horizontal black), GL04V04ACC‐1 (vertical white), GL04V04ACC‐3 (vertical black). Programming and commissioning are excluded. MANUFACTURER: EELECTRON – ART. GP04A04KNX |  |
| pcs | 1 | GP08A04KNX | KNX switch GlassPad KNX 8 CH. / 4 Modules KNX capacitive switch 8 channels available in 2 colours (white and black). Mounting in 4 modules inwall box. The product has led signalling for each channels and can be installed vertically or in horizontal. Pression on the switch can be long or short and it can manage scenarios or sequences. The customization of the glass color, texts and icons for residential and building filed is available on demand. The products if fully programmable with ETS. This device must be combined with glass cover for GlassPad, cod. GL08H04ACC‐1 (horizontal white), GL08H04ACC‐3 (horizontal black), GL08V04ACC‐1 (vertical white), GL08V04ACC‐3 (vertical black). Programming and commissioning are excluded. MANUFACTURER: EELECTRON – ART. GP08A04KNX |  |
| pcs | 1 | GT00A03KNX | KNX CAPACITIVE THERMOSTAT The inwall Glass Thermostat GT00A03KNX is a KNX wall mounting device designed for HVAC applications in Home and Building installations (e.g., offices, hospitals, hotels, private houses, etc..). The Glass on the front side displays the following information: actual temperature in °C, actual setpoint, Fan coil speed, HVAC mode status, heating and cooling status, status of controlled heating / cooling equipment. Control elements available on the front are: 1 capacitive button to increase the temperature setpoint, 1 capacitive button to decrease the temperature setpoint, 1 capacitive button to set the HVAC mode, 1 capacitve button to set heating or cooling mode. GT00A03KNX is intended to be used in rectangular 3 module box mounted in vertical. Device is equipped with KNX communication interface. Auxiliary power supply 24V DC. Glass plate dimensions: 76 x 106 mm MANUFACTURER: EELECTRON – ART. GT00A03KNX |  |
|  |  | MiniSWITCH KNX |  |  |
| pcs | 1 | WB10AxxKNX | KNX inwall SWITCH 1 channel KNX switch suitable with plates of Eelecta’s range 2 or 4 modules. The range is composed by 3 models that include 1,2 or 4 channels switches with a dimension of 45x45 mm (frame not included). For each channel are available 2 signaling led's (white/blue). The switches are available in 2 different colours (white or anthracite) and the plates in 3 colours (white, silver e black). Programming and commissioning are excluded. MANUFACTURER: EELECTRON – ART. WB10A10KNX (WHITE VERSION) MANUFACTURER: EELECTRON – ART. WB10A30KNX (BLACK VERSION) |  |
| pcs | 1 | WB20AxxKNX | KNX inwall SWITCH 2 channels KNX switch suitable with plates of Eelecta’s range 2 or 4 modules. The range is composed by 3 models that include 1,2 or 4 channels switches with a dimension of 45x45 mm (frame not included). For each channel are available 2 signaling led's (white/blue). The switches are available in 2 different colours (white or anthracite) and the plates in 3 colours (white, silver e black). Programming and commissioning are excluded. MANUFACTURER: EELECTRON – ART. WB20A10KNX (WHITE VERSION) MANUFACTURER: EELECTRON – ART. WB20A30KNX (BLACK VERSION) |  |
| pcs | 1 | WB40AxxKNX | KNX inwall SWITCH 4 channels KNX switch suitable with plates of Eelecta’s range 2 or 4 modules. The range is composed by 3 models that include 1,2 or 4 channels switches with a dimension of 45x45 mm (frame not included). For each channel are available 2 signaling led's (white/blue). The switches are available in 2 different colours (white or anthracite) and the plates in 3 colours (white, silver e black). Programming and commissioning are excluded. MANUFACTURER: EELECTRON – ART. WB40A10KNX (white) / ART. WB40A30KNX (black) |  |